Section C  Developing Your Skills

Targeted Writing Exercise

Exercise One: use, excavated soil, back-filling

Answer: Excavated soil shall be used for back-filling.

Exercise Two: use, tandem drum roller, compaction

Answer: Tandem drum rollers shall be used for compaction.

Exercise Three: plastering material, technical specification

Answer: Plastering material shall be as per the technical specification.

Exercise Four: curing period for concrete, 7 days

Answer: The curing period for concrete shall be a minimum of 7 days.

Section D Expanding Your Creativity

Draft the methodology and sequence of work in a construction method statement for asphalt base course/binder course（沥青基层/粘层）

- thickness of bituminous base course, as per, approved shop drawing and project specification

- lay, bituminous material, paver

- achieve, compaction of the bituminous material, steel wheel tandem vibratory rollers and pneumatic tire rollers

- carry out, initial compaction or breakdown rolling, a tandem vibratory roller, operating in the static mode

- carry out, subsequent compaction, tandem vibratory rollers, operating in the vibratory mode, followed by, pneumatic tire rollers, to achieve the required degree of compaction, project specification

- carry out, a final “finishing” roll, a tandem vibratory roller, operating in the static mode, to remove any remaining roller marks

Answer:

- The thickness of bituminous base course shall be as per the approved shop drawing and project specification.

- 沥青基层的厚度应符合经批准的施工图和项目规范。

- Pavers shall be used to lay the bituminous material.

- 应使用摊铺机来铺设沥青材料

- Steel wheel tandem vibratory rollers and pneumatic tire rollers shall be used to achieve compaction of the bituminous material.

- 应使用双钢轮振动压路机和轮胎式压路机来进行沥青材料的压实。

- A tandem vibratory roller operating in the static mode shall be used to carry out initial compaction or breakdown rolling.

- 应使用双钢轮振动压路机的静压模式，进行初始压实或分解碾压。

- Tandem vibratory rollers operating in the vibratory mode followed by pneumatic tire rollers shall be used to carry out subsequent compaction to achieve the required degree of compaction as per project specification.

应采用双钢轮振动压路机的振动模式来压实，然后使用轮胎式压路机进行后续压实，以达到项目规范所需的压实程度。

- A tandem vibratory roller operating in the static mode shall be used to carry out a final “finishing” roll to remove any remaining roller marks.

- 应使用双钢轮振动压路机的静压模式进行最终的“修整”压实，以去除残余的滚筒痕迹。